Tech Inspection

Onsite technical inspection is a brief inspection of the vehicle prior to driver going to the registration table. Volunteers are needed to complete several tasks in order to go through the 40-80 cars in an hour and a half. Setup

Two lines of cars

Teams of 2-5 people working each line (Chief of Tech has final say)

As driver pulls up ask them to blip the throttle to check for throttle return and step on brakes to check brake lights

Items to check under hood

Obvious leaking fluids

Brake fluid color and level

Stripped/loose electrical wires/connections

Oil/powersteering/brake fluid caps tight

Battery secured (if in engine compartment)

Items to check outside car

Brake linings >thickness of the backing plate

Shake all four wheels to check wheel bearings/tie rods/ball joints

Tires for adequate tread (no cording, chunking, rubbing on fender or suspension, no snow tires)

No body parts loose, general safety appearance of the vehicle

Tow hooks installed

Window tint not overly dark (must be able to easily see through windows from outside)

Gas door and cap shut/tight

Exterior cameras must be securely hardmounted, NO VACUUM SUCTION MOUNTS WITHOUT TEATHERS

Items to check inside car

Floormats removed

Battery secured (if in trunk)

All loose items removed (radar detector, ez pass, gps, pens, coins, etc.)

Cameras must be hard mounted (suction cups may be used if camera is tethered adequately and limits the movement from reaching driver or passenger)

Seats/restraints are equal for both driver and passenger

Harnesses (if used) are routed correctly Helmet Check

Helmet (must be Snell rated for current or one previous rating year, Snell sticker found on the inside of the helmet under the lining—For this event, Snell 2005/2010 –SA or M) Apply helmet inspection sticker if valid.

Passed?

Apply tech inspection sticker to upper drivers side corner of windshield and stamp pre-event tech form—then hand form to Tech Chief